FACTORIAL OF A GIVEN NUMBER

EXP NO: 9

AIM: To find the factorial of a given number using 8085 microprocessor.

ALGORITHM:

- Load the data into register B

- To start multiplication set D to 01H

- Jump to step 7

- Decrements B to multiply previous number

- Jump to step 3 till value of B>0

- Take memory pointer to next location and store result

- Load E with contents of B and clear accumulator

- Repeatedly add contents of D to accumulator E times

- Store accumulator content to D

- Go to step 4

PROGRAM:

LDA 2001

MOV B,A

MVI C,#01

MVI E,#01

LOOP: MOV D,C

MVI A,00H

LP: ADD E

DCR D

JNZ LP

MOV E,A

INR C

DCR B

JNZ LOOP

MOV A,E

STA 2010

HLT

INPUT:

OUTPUT:

RESULT: Thus the program was executed successfully using 8085 processor simulator.

LARGEST NUMBER IN AN ARRAY

EXP NO: 10

AIM: To find the largest number from an array using 8085 processor.

ALGORITHM:

- Load the address of the first element of the array in HL pair.

- Move the count to B register.

- Increment the pointer.

- Get the first data in A register.

- Decrement the count.

- Increment the pointer.

- Compare the content of memory addressed by HL pair with that of A register.

- If carry=0, go to step 10 or if carry=1 go to step 9

- Move the content of memory addressed by HL to A register.

- Decrement the count.

PROGRAM:

LXI H,2050

MOV C,M

DCR C

INX H

MOV A,M

LOOP1: INX H

CMP M

JNC LOOP

MOV A,M

LOOP: DCR C

JNZ LOOP1

STA 2058

HLT

INPUT:

OUTPUT:

RESULT: Thus the program was executed successfully using 8086 processor simulator.

SMALLEST NUMBER IN AN ARRAY

EXP NO: 11

AIM: To find the smallest number from an array using 8085 processor.

ALGORITHM:

- Load the address of the first element of the array in HL pair.

- Move the count to B register.

- Increment the pointer.

- Get the first data in A register.

- Decrement the count.

- Increment the pointer.

- Compare the content of memory addressed by HL pair with that of A register.

- If carry=1, go to step 10 or if carry=0 go to step 9

- Move the content of memory addressed by HL to A register.

- Decrement the count.

PROGRAM:

LXI H,2050

MOV C,M

DCR C

INX H

MOV A,M

LOOP1: INX H

CMP M

JC LOOP

MOV A,M

LOOP: DCR C

JNZ LOOP1

STA 2058

HLT

INPUT:

OUTPUT:

RESULT: Thus the program was executed successfully using 8085 processor simulator.

ASCENDING ORDER

EXP NO: 12

AIM: To compute ascending order of an array using 8085 processor.

ALGORITHM:

- Initialize HL pair as memory pointer.

- Get the count at memory and load it into C register

- Copy it in D register (for bubble sort (N-1)) times required).

- Get the first value in A register.

- Compare it with the value at next location.

- If they are out of order, exchange the contents of A register and memory.

- Decrement D register content by 1

- Repeat step 5 and 7 till the value in D register become zero.

- Decrement the C register content by 1.

- Repeat steps 3 to 9 till the value in C register becomes zero.

PROGRAM:

LOOP: LXI H,3500

MVI D,00

MVI C,05

LOOP1: MOV A,M

INX H

CMP M

JC LOOP2

MOV B,M

MOV M,A

DCX H

MOV M,B

INX H

MVI D,01

LOOP2: DCR C

JNZ LOOP1

MOV A,D

RRC

JC LOOP

HLT

INPUT:

OUTPUT:

RESULT: Thus the program was executed successfully using 8085 processor simulator.

DESCENDING ORDER

EXP NO: 13

AIM: To compute descending order of an array using 8085 processor.

ALGORITHM:

- Initialize HL pair as memory pointer.

- Get the count at memory and load it into C register

- Copy it in D register (for bubble sort (N-1)) times required).

- Get the first value in A register.

- Compare it with the value at next location.

- If they are out of order, exchange the contents of A register and memory.

- Decrement D register content by 1

- Repeat step 5 and 7 till the value in D register become zero.

- Decrement the C register content by 1.

- Repeat steps 3 to 9 till the value in C register becomes zero.

PROGRAM:

LOOP: LXI H,3500

MVI D,00

MVI C,05

LOOP1: MOV A,M

INX H

CMP M

JNC LOOP2

MOV B,M

MOV M,A

DCX H

MOV M,B

INX H

MVI D,01

LOOP2: DCR C

JNZ LOOP1

MOV A,D

RRC

JC LOOP

HLT

INPUT:

OUTPUT:

RESULT: Thus the program was executed successfully using 8085 processor simulator.

ADDITION OF N NUMBERS

EXP NO: 14

AIM: To compute addition of N numbers using 8085 processor.

ALGORITHM:

- Load the base address of the array in HL register pair.

- Load the memory with data to be added.

- Take it as count.

- Initialize the accumulator with 00.

- Add content of accumulator with content of memory.

- Decrement count.

- Load count value to memory location.

- Repeat step 5.

- Check whether count has become 0.

- Halt.

PROGRAM:

LXI H,8000

MOV C,M

MVI A,00

MOV B,A

LOOP: ADD C

JNC SKIP

INR B

SKIP: DCR C

JNZ LOOP

LXI H,8007

MOV M,A

INX H

MOV M,B

HLT

INPUT:

OUTPUT:

RESULT: Thus the program was executed successfully using 8085 processor simulator.

SWAPPING OF NUMBERS

EXP NO: 15

AIM: To compute swapping of numbers using 8085 processor.

ALGORITHM:

- Load a 8-bit number from memory location into accumulator.

- Move value of accumulator into register H.

- Load a 8-bit number from next memory location into accumulator.

- Move value of accumulator into register D.

- Exchange both the registers pairs.

- Halt

PROGRAM:

LDA 2001

MOV B,A

LDA 2002

MOV C,A

STA 2003

MOV A,B

STA 2004

HLT

INPUT:

OUTPUT:

RESULT: Thus the program was executed successfully using 8085 processor simulator.

SQUARE OF NUMBER

EXP NO: 16

AIM: To compute square of number using 8085 processor.

ALGORITHM:

- Load the base address of the array in HL register pair.

- Assign accumulator as 0.

- Load the content of memory location specified into register.

- Add content of memory location with accumulator and decrement register content by 01.

- Check if register holds 00, if so store the value of accumulator in memory location.

PROGRAM:

LXI H,8000

XRA A

MOV B,M

LOOP: ADD M

DCR B

JNZ LOOP

STA 8001

HLT

INPUT:

OUTPUT:

RESULT: Thus the program was executed successfully using 8085 processor simulator.